CHECKLIST

- Is trailer coupled correctly, i.e. lock catch down, ball fully engaged, braking safety strap attached securely to tow vehicle.

- Handbrake off when hooked up and on when disconnecting from car.

- The jockey wheel has a locked position when fully wound up. Please ensure it is wound down before attempting to push trailer as failure to do so will result in a bent jockey arm and flat tyre. If unsure of this ask an experienced tower. When attached to vehicle ensure the whole body of the jockey wheel is as high as possible from the road and pulled just above the body of the trailer.

- All straps are secure and tails tied securely to body of trailer. Do not overtighten as this can damage the boat.

- When finished and returned to storage just loosen straps to relieve pressure on the boat.

- Rear Legs up/down depending on whether loading or unloading

- When unplugging light plug do not pull out by the lead, only use the body of the male connector and hold the body of the female connector whilst doing so

- Lights plugged in and all working correctly

- Nose weight – ensure boat is loaded with slightly more of an emphasis on weight towards front or ideally over axle. If nose has to be pushed down to connect to tow bar it is incorrectly loaded.

- Tyres, check pressures including spare

- Toolboxes – These are attached to the trailers and are not to be removed for any reason. The content can be taken out in situ. Please ensure all contents are in the boxes for breakdown. These are listed in the lid of the box. It is the towers responsibility to ensure it is all there and fastened securely. Please report immediately if anything is missing.

- Breakdown – Please use triangle provided in toolbox and wear yellow vest. Call RAC/AA if member. If not stay (safely) with/near vehicle and call for assistance. Within reason the club will help with cost of this but we strongly suggest you get suitable roadside assistance cover.

- Puncture – It is very difficult to change a wheel on the trailer if you have a puncture. If possible and tyre has not shredded please use the supplied tin of puncture repair and top up with air. Follow instructions on the can especially relating to speed.

- Be very aware of people and objects around you

PLEASE, PLEASE, PLEASE REPORT ANY FAULTS IMMEDIATELY